NahidW 3D Face Scan Pipeline

Complete Workflow Documentation

From raw scanner TIF to landmark-aligned face images

# Contents

# 1. Overview

This pipeline processes raw 3D face scans from a Cyberware scanner into a series of consistently aligned point clouds and 2D landmark-aligned images. It is designed for longitudinal scar monitoring — comparing a patient's face at different time points (Day 0, Day 28, etc.).

The pipeline runs entirely from the command line. Each step is a standalone Python script that reads from one folder and writes to another. No step modifies its input files.

## 1.1  High-level flow

The pipeline has 7 steps organized into 3 phases:

| Phase | Purpose |
| --- | --- |
| Phase 1 Pre-orient | Fix yaw rotation so every scan faces forward (+Z). Uses the geometric nose tip as reference. Saves corrected scans as new files — inputs are never overwritten. |
| Phase 2 ICP Alignment | Scale normalisation + multi-scale Iterative Closest Point registration. Aligns all scans for a patient to a common Day 0 reference frame. Final bindi-snap step locks nose tips exactly. |
| Phase 3 Landmark Alignment | Render 2D frontal views from the ICP-aligned 3D scans. Detect 5 anatomical landmarks. Apply scale + translation to produce standardised 2D face images. |

# 2. Folder Structure

## 2.1  Input (scanner output)

D:/NahidW/ Dataset/ 3d_scans/ ply/ pat1/ pat1day0C_aligned.ply ← raw scanner output pat1day28A_aligned.ply pat1day28C2_aligned.ply pat2/ pat2day0A_aligned.ply ...

## 2.2  After running the full pipeline

D:/NahidW/ Dataset/ 3d_scans/ply/pat1/ pat1day0C_aligned.ply ← original (untouched) pat1day0C_aligned_oriented.ply ← yaw-corrected [pre_orient.py] pat1day0C_aligned_oriented_icp.ply ← ICP-registered [face_icp_nahid.py] pat1day28A_aligned_oriented_icp.ply pat1day28C2_aligned_oriented_icp.ply pat1_merged_icp.ply ← all 3 scans merged frankfurt_aligned/pat1/ pat1day0C_aligned_oriented_icp_front.png ← frontal render [step1_render.py] pat1day28A_aligned_oriented_icp_front.png pat1day28C2_aligned_oriented_icp_front.png landmarks_raw/pat1/ pat1day0C_aligned_oriented_icp_landmarks.json ← [step1_detect_landmarks.py] pat1day0C_aligned_oriented_icp_landmarks.png ... landmark_aligned/pat1/ pat1day0C_aligned_oriented_icp_aligned.png ← final output [step2_align_images.py] pat1day0C_aligned_oriented_icp_aligned_preview.png ...

# 3. Step-by-Step Pipeline

| Step 1 Pre-orient scans |  |
| --- | --- |
| Script pre_orient.py What it does Detects geometric nose tip (top 2% Z in central face strip). Measures yaw offset from +Z axis. Rotates each scan around its Y axis to cancel that offset. Saves corrected scans as new files — originals untouched. | Reads 3d_scans/ply/pat1/*_aligned.ply Writes 3d_scans/ply/pat1/*_aligned_oriented.ply |

Key detail: only the Y-axis (yaw / horizontal) rotation is corrected. Tilt and roll are left for ICP. Works for any patient — purely geometric, no color detection.

| Step 2 ICP alignment |  |
| --- | --- |
| Script face_icp_nahid.py What it does 1. Cleans scatter noise below Y=100. 2. Measures 3D geometric scale (X/Y/Z span). 3. Translates source centroid onto reference centroid. 4. Runs 5-scale point-to-plane ICP (40→15→10→5→2 mm). 5. Final bindi-snap: detects red nose mark, translates to lock nose tips exactly. | Reads 3d_scans/ply/pat1/*_aligned_oriented.ply Writes 3d_scans/ply/pat1/*_aligned_oriented_icp.ply 3d_scans/ply/pat1/pat1_merged_icp.ply 3d_scans/ply/alignment_report.csv |

Reference scan: the earliest day (Day 0) for each patient. All later scans register to Day 0. The Day 0 scan is also saved with the _icp suffix (identity transform) so all files are consistent.

| Step 3 Render front view |  |
| --- | --- |
| Script frankfurt_step1_render.py What it does Opens each ICP-aligned PLY in Open3D. Positions camera at +Z (Y=up) to get a true frontal view matching the ICP coordinate system. Fills point cloud gaps with inpainting and brightens for MediaPipe detection. | Reads 3d_scans/ply/pat1/*_aligned_oriented_icp.ply Writes frankfurt_aligned/pat1/*_aligned_oriented_icp_front.png |

The camera direction is fixed: front=[0,0,1], up=[0,1,0]. This works because pre_orient already rotated all scans to face +Z, so no yaw detection is needed here.

| Step 4 Detect landmarks |  |
| --- | --- |
| Script step1_detect_landmarks.py What it does Runs MediaPipe FaceLandmarker on the rendered front PNG. Detects 5 landmarks: sellion (nose bridge), pronasale (nose tip), menton (chin), l_tragion (left ear), r_tragion (right ear). Falls back to interactive manual click window if MediaPipe fails. | Reads frankfurt_aligned/pat1/*_oriented_icp_front.png Writes landmarks_raw/pat1/*_landmarks.json landmarks_raw/pat1/*_landmarks.png |

Manual fallback usage:

python step1_detect_landmarks.py --manual pat1day28A_aligned_oriented_icp

| Step 5 Align to template |  |
| --- | --- |
| Script step2_align_images.py What it does Computes a scale + translate transform (no rotation — faces already front-facing from pre_orient) that maps the detected landmark positions to fixed template positions on a 600x700 canvas. Saves clean aligned PNG and a preview with landmark dots. | Reads landmarks_raw/pat1/*_landmarks.json frankfurt_aligned/pat1/*_front.png Writes landmark_aligned/pat1/*_aligned.png landmark_aligned/pat1/*_aligned_preview.png |

Template positions (pixels on 600x700 canvas):

| Landmark | X | Y |
| --- | --- | --- |
| sellion (nose bridge) | 300 | 180 |
| pronasale (nose tip) | 300 | 270 |
| menton (chin) | 300 | 440 |
| l_tragion (left ear) | 145 | 235 |
| r_tragion (right ear) | 455 | 235 |

# 4. Quick Reference

## 4.1  Run order

Run from the Coding folder. All scripts auto-detect the NahidW root.

python pre_orient.py

python face_icp_nahid.py

python frankfurt_step1_render.py --overwrite

python step1_detect_landmarks.py

python step2_align_images.py --show

## 4.2  Per-patient override

python pre_orient.py --patient pat1

python face_icp_nahid.py --patient pat1

python step1_detect_landmarks.py --patient pat1

## 4.3  Full summary table

| Step | Script | Input | Output |
| --- | --- | --- | --- |
| 1 | pre_orient.py | *_aligned.ply | *_aligned_oriented.ply |
| 2 | face_icp_nahid.py | *_aligned_oriented.ply | *_aligned_oriented_icp.ply |
| 3 | frankfurt_step1_render.py | *_aligned_oriented_icp.ply | *_oriented_icp_front.png |
| 4 | step1_detect_landmarks.py | *_oriented_icp_front.png | *_landmarks.json / .png |
| 5 | step2_align_images.py | *_landmarks.json + *_front.png | *_aligned.png |

# 5. Key Configuration Parameters

All configurable values are at the top of each script. No hardcoded values elsewhere.

## pre_orient.py

| NOSE_X_HALF | 60 | Half-width (mm) of central X strip for nose tip search |
| --- | --- | --- |

| NOSE_Y_MIN | 280 | Minimum Y (mm) — above mouth even when fully open |
| --- | --- | --- |

| NOSE_Z_TOP_PCT | 98 | Use top N% of Z to find nose tip (robust to outliers) |
| --- | --- | --- |

## face_icp_nahid.py

| NOISE_Y_MIN | None (auto) | Auto-detect scatter floor from histogram |
| --- | --- | --- |

| ICP_SCALES | 40→20→10→5→2 mm | 5 ICP passes coarse→fine |
| --- | --- | --- |

| MARKER_R_MIN | 0.5 | Red channel threshold for bindi detection |
| --- | --- | --- |

# 6. Important Notes

## 6.1  Files are never overwritten

Every script writes new files with a new suffix. The original *_aligned.ply scanner outputs remain untouched throughout the entire pipeline. If you need to re-run, use --overwrite or --redetect flags.

## 6.2  Bindi snap

The ICP pipeline includes a final nose-tip snap step that detects the red bindi mark by color (R > 0.5, G < 0.3, B < 0.3) and applies a pure translation to align nose marks exactly. If a patient has no visible red marker, this step is automatically skipped and the ICP result is kept as-is.

## 6.3  Manual landmark fallback

If MediaPipe fails to detect a face (low texture, burn scars, unusual expression), step1_detect_landmarks.py opens an interactive window. Click the 5 landmarks in order. Right-click to undo. Results are cached as JSON so you only click once per scan.

## 6.4  Coordinate system (ICP files)

All *_aligned_oriented_icp.ply files share a consistent coordinate system:

- Y axis = vertical (superior/inferior). Face runs from Y≈100 (chin) to Y≈456 (forehead top).

- Z axis = depth. Positive Z = front of face (nose points toward +Z).

- X axis = left/right. Face centre near X=0.

NahidW Face Scan Pipeline  —  Internal Documentation